10T Right-hand Drive Four-Wheel Drive chassis

Chassis Brief configuration:

Cab: Single row, right-hand drive, 2080mm wide

Wheelbase: 3390mm

Chassis dimensions: 6080*2170*2580mm

Drive configuration: 4*4

Engine: Yunnei YN38CRD1, inline four-cylinder, 3.76L displacement, China IV emission standard, 95kw, 365N.M

Transmission: Fast 6DSX60T (6-speed manual)

Front axle: 2.7T, drum type (four-wheel drive front axle)

Rear axle: 5.5T, drum axle, 4.33 gear ratio

Frame: 188 double layer * 800mm, straight beam

Leaf springs: 8/10+7

Tires: 245/70R19.5 or 8.25R20, 6+1 tires

Transfer case: Hubei Senyu

Low voltage platform: 24V

Fuel tank: 90L iron

Braking: Air brakes, exhaust brakes, exhaust brakes, ABS

Other features: Air conditioning (heating and cooling), power windows, multi-function steering wheel

Front/rear track: 1760/1630mm

Chassis front/rear overhang: 1265/1425mm

Chassis approach angle/departure angle: 25°/18°

Minimum ground clearance: 200mm

Vehicles that can be modified : logistics vehicles, sanitation vehicles , etc.